Marshall SDK

Java

─
Document version: 3.0.1
Update Date: 11/2021

Mickey Atias

Nayax / Bitmick

Einstein 3,
Hezliya

| Author | Date | Version | Change |
| --- | --- | --- | --- |
| Mickey Atias | 4/11/2018 | 2.1.0 | Added get_transaction_id API |
| Yael Schwartz | 11/11/2021 | 3.0.1 | Edited the document |

Contents

0

Overview	3

Terms	3

Quick-Start	4

The Vending Process	5

API	6

Framework	6

API	6

Link	7

Events	7

Structures	7

Vend	9

Events	9

Structures	10

APIs	11

11

Socket	13

General Marshall Facilities	14

Events	14

API	14

Overview

This Document describes the structure, methods and API for the Marshall Java SDK.

Terms

| Device | Nayax’ payment and communication Device. e.g. Amit / VPOS Touch / Onyx / … The device is able to communicate with DCS only! |
| --- | --- |
| Peripheral | - Vending Machine - EV Charger - Lockers |
| VMC | Vending Machine Controller - The host which runs the Marshall SDK and is connected via serial interface (RS232) to the cashless device. A type of peripheral. |
| Cashless | The device that reads Credit-cards / Prepaids |
| MDB | Multi-Drop-Bus protocol |
| Session | Vending process session |
| POS | Point of Sale |
| Mutli-Session | A feature of the Marshall protocol - allows several credit-sessions to remain opened, until a final price is calculated. Relevant for example in cases of time-based vending machines. A transaction working in two separate parts: 1) Pre-authorization 2) Settlement Suits EV Chargers, Smart Lockers etc. where final product price is either unknown or transaction may last longer than 18 hours (65535 seconds). - Transactions can stay "open" up to 23 hours. - Up to 60 transactions can be active simultaneously. - According to Nayax’ policy, Authorized transactions with no Settlement are automatically settled after 24 hours. |
| Multi-Vend | The ability to sell multiple items (more than one type and more than 1 per type) in one vending session. A shopping cart feature. Suits vending machines which can dispense multiple products in one transaction e.g. Smart Fridges. |
| Session Timeout (up to 1380 minutes) | The amount of time from starting a session util it's timeout. By default, a multi-session timeout is set to 60 minutes. In case a transaction needs to be open for longer, Nayax needs to change this attribute accordingly. |
| Reader Always On | The reader is never Disabled by the Peripheral and a transaction may start by presenting a card. Suits peripherals requiring to present card first in order to start the service, e.g. Smart Fridges where a successful card presentation opens the door. |
| Always Idle | Transaction is triggered by selecting first a product on the machine. No need in the START button or presenting a card first to the device. In MDB translation: no need for the device to send the “Begin Session” – the machine will directly send the “Vend Request” to launch a transaction. |

Quick-Start

Using the Marshall JAVA SDK is very straight-forward. Just follow the following steps:

- Import the Code / . JAR file into your project

- In your code, import the following:
import com.bitmick.marshall.models.*;

- Instantiate a vmc configuration object:
public vmc_configuration vmc_config = new vmc_configuration();

- Get the vmc instance:
public vmc_framework vmc_instance;
vmc_instance = vmc_framework.getInstance();

- Fill in the appropriate fields. For example, standard configuration looks like this:

vmc_configuration vmc_config = new vmc_configuration();

vmc_config.port_vpos = "COM17"; // see explanation bellow about the COM port
vmc_config.port_vpos_baud = 115200;

vmc_config.machine_type = vmc_configuration.machine_type_none;
vmc_config.model = "my_machine_model";
vmc_config.serial = "01234567";
vmc_config.sw_ver = "0.0.0.1";

vmc_config.multi_session_support = false;
vmc_config.price_not_final_support = false;
vmc_config.reader_always_on = false;
vmc_config.always_idle = false;

vmc_config.mifare_approved_by_vmc_support = false;
vmc_config.mag_card_approved_by_vmc_support = false;

vmc_config.dump_packets_level = vmc_configuration.debug_level_dump_all;

vmc_config.debug = true;

vmc_instance.configure(vmc_config);

vmc_instance.link.set_events(new vmc_link_callback());

COM port- make sure that you change the COM to the one your Marshall cable is connected to!

- Apply the serial interface driver
(a reference implementation is provided, and is good for almost all peripherals):
vmc_instance.link.set_lowlevel(new lowlevel_serial());

Note: if using Android platform- you must use an Android driver (from your provider). For consultation- you can contact Nayax.

- Register the callback events, such as vending events class
(you, handling the various vending events. see the sample code)
vmc_instance.vend.register_callbacks(this);
vmc_instance.socket.register_callbacks(new vmc_socket_events());
vmc_instance .general.register_callbacks(new time_update_callback ());

- Start the SDK, and wait for the onReady() event:
vmc_instance .start();

The Vending Process

Once the SDK is running, and the link with the VPOS is established (onReady() was called)
the VPOS is ready for the vending / cashless process. Here is described the simple Credit-Card / Pre-Paid single session process:

- Enable the device to accept Credit-Cards / Prepaid cards / etc
m_vmc.vend.session_start(vmc_vend_t.session_type_credit_e);

- Once a client presented a card, and the card has been approved, the onSessionBegin()
Event will be fired. This is the time for the peripheral to ask for payment approval:
 // prepare session object
m_session = new vmc_vend_t.vend_session_t(2000, 1, (byte) 0, 10);

(Note: for multi-vend session, just turn on the multi_vend_support flag. In that case, the session would include a list of products (as appearing in ArrayList<vmc_vend_t.vend_item_t> list )

// vend request
m_vmc.vend.vend_request(m_session);

- After the VPOS approves the vend request, the peripheral needs to notify the SDK
that it has successfully delivered the product to the client. This is done by implementing the onVendApproved(...) API and returning 
    True: for successful vend process. In this case the client’s credit-card will be 
              charged with the requested amount of money
    False: for failed vend process

- In case multi-session configuration is applied, when the session on the peripheral
is finished, user needs to call m_vmc.vend.session_close(m_session)

- In case multi-session, wait for public void onSettlement(boolean success) event

- In any case - single session or multi-session, onReady() will be called upon
finishing the vending process

API

Framework

// get framework instance

vmc_framework m_vmc  = vmc_framework.getInstance();

API

public void start();

Start the SDK thread

public void stop()

Stop the SDK thread

Link

Events

public interface vmc_link_events_t

void onReady(vmc_link.vpos_config_t config);
void onCommError();

Structures

public class vmc_configuration

public Object port_vpos;
public int port_vpos_baud;

public String serial;
public String model;
public String sw_ver;
public int    machine_type;

public boolean reader_always_on;
public boolean multi_session_support;
public boolean multi_vend_support;
public boolean always_idle;
public boolean explicit_vend_success;
public boolean price_not_final_support;
public boolean mag_card_approved_by_vmc_support;
public boolean mifare_approved_by_vmc_support;

public boolean debug;
public int dump_packets_level;

Note- more in-depth explanation about each of those flags' meaning under "SDK's flags description"

public static class vpos_config_t

// marshall protocol version, msb
public byte   prot_ver_major;
// marshall protocol version, lsb
public byte   prot_ver_minor;
public byte   language;
// country code, iso<t.b.d>
public byte[] country_code;
// currency code, based on same iso<t.b.d> above
public byte[] currency_code;
// how price is represented, as 10 exponent. for example:
// if decimal place is 2 and price requested is 100, then the actual price is 1.00 (decimal places: 2 positions)
// if decimal place is 0 and price requested is 100, then the actual price is 100.
// all pricing is according to currency configured on DCS (above examples, 1.00$, and 100$ respectivly)
public byte   decimal_place;
// byte array representing vpos serial number
public byte[] vpos_serial;
// machine type. see vmc_configuration.machine_type_xxxxxx
public int    machine_type;
// merchant id
public byte[] merchant_id;
// merchant teminal id
public byte[] acquirer_terminal_id;
// machine location
public byte[] machine_location;

APIs

public vmc_link set_events(vmc_link_events_t events);

public void start();

public void stop();

Vend

Events

public abstract static interface vend_callbacks_t

// called when peripheral framework is ready to start session (after a link has been established or a previouse session has ended)
public abstract void onReady(vmc_link.vpos_config_t config);

// called when a client presents a credit-card
public abstract void onSessionBegin(int funds_avail);

// on event of received data
public abstract void onTransactionInfo(vend_session_data_t data);

// the vending request has been approved
public abstract boolean onVendApproved(vend_session_t session);

// the vending request has been denied
public abstract void onVendDenied(vend_session_t session);

// transaction settlement
public abstract void onSettlement(boolean success);

// called when the peripheral receives "Status" command from Nayax's device (Marshall Protocol section 8.11)

Public abstract void onStatus(int status);

// called when the peripheral gets "Get Session Status" command from Nayax's device, in order to know which sessions are open (Marshall Protocol section 8.27.5). Very useful in cases the peripheral software restarts and we want to know which sessions are still opened. Only relevant for Multi-session mode

Public abstract void onOpenedSessions(ushort[] sessions);

// the response to "Get_reader status" is "Reader status". This function returns enabled/disabled (Marshall Protocol section 8.27.9)

Public abstract void onReaderState(bool enabled);

// a DCS option to enable a QRcode receipt. Used in order to pass to the peripheral the QR code generated by the eReceipt utility. The peripheral will display this QR code on its display to enable the consumer to scan the eReceipt on the mobile phone. (Marshall Protocol section 8.12)

Public abstract void onReceipt(int ereceipt_type, String data);

Structures

Product request instance

public static class vend_item_t

// product code (aka mdb code)
public short code;
// unit price (real price is price / (10 ^ decimal places)
public int   price;
// quantity of product. engine will multiply price * qty
public int   qty;
// unit of measure (0: don't care, a discrete unit (such as single bottle of drink)
public byte  unit;

public static class vend_session_data_t

public long transaction_id;
public short choose_product_timeout;
public int card_type;
public int card_entry_mode;
public String card_bin;
public String card_bin_hash;
public String prop_card_uid;
public int vmc_auth_status;
public int com_status;
public String excel_data;
public String cc_last_4_digits;

Vending Session Handle

public static class vend_session_t

public int funds_avail;

// user should set this, usually  in case user cancels or failed to dispense. please look at session_status_xxxxxxx
public int session_status;

// session data – returned from VPOS upon valid session
public vend_session_data_t data;

// products list. Note- On multi-session only single product is allowed (the list is not being used)
public ArrayList<vend_item_t> products_list;

Events

public abstract static interface vend_callbacks_t

// called when peripheral framework is ready to start session (after a link has been established or a previous session has ended)
public abstract void onReady(vmc_link.vpos_config_t config);

// called when a client presents a credit-card
public abstract void onSessionBegin(int funds_avail);

// on event of received data
public abstract void onTransactionInfo(vend_session_data_t data);

// the vending request has been approved
public abstract boolean onVendApproved(vend_session_t session);

// the vending request has been denied
public abstract void onVendDenied(vend_session_t session);

// transaction settlement
public abstract void onSettlement(boolean success);

//called when "Status" is received. Returns the status of a session (settlement/timeout/unexpected error, etc) as explained in the Marshall Protocol section 8.11

public abstract void onStatus(int status);

//called when "Session status" is received

public abstract void onOpenedSessions(short[] sessions);

//called when "Reader status" is received

public abstract void onReaderState(bool enabled);

//called when "QR Code" is recieved

public abstract void onReceipt(int ereceipt_type, string data);

APIs

public void register_callbacks(vend_callbacks_t callbacks);

Register peripheral application for vending events. 
See above description for vend_callbacks_t

public boolean is_ready();

Return true if a peripheral and device link is established

public void session_start(int session_type);

Enable the device to be ready to accept Credit-Cards / Prepaid.
The session_type defines what happens when the user presents a card:

public static final int session_type_credit_e = 0x00;
public static final int session_type_info_e = 0x01;

In info session, no actual money transfer is involved. Just the payment medium information
Is transferred to the peripheral application, via onTransferData

public void session_modify(int session_type);

Allows the peripheral application to modify the session type after the reader has been enabled,
Instead of cancelling, and re-starting.

See session_type_… above

public int get_session_type();

Returns the current session type the device is enabled to

public void session_cancel();

Stops the device's reader from waiting for payment medium

public void session_close(vend_session_t session);

In case peripheral application is configured as multi-session, every transaction needs to be closeds
In order to make an actual money charge. The session object that was  created on vend_request
Should be assigned into session_close

public void vend_request(vend_session_t session);

After a payment medium is read (Credit-Card / Pre-Paid / etc), and authorized,
The user should call the vend_request API, in order to charge payment
from the customer (see vend_session_t for more information)
Note: it is possible to request vending of more than one product (multi-vend).
Just add vend_item_t object to  products_list inside session.

Note: no more than 20 different products, in case of multi-vend

public void vend_cash_sale(vmc_vend_t data);

Peripheral application can notify the device of a cash sale. This data is delivered
To the backend, for statistics purposes and so on.

public void vend_pp_pay(String pp_number);

In case of previously reading pre-paid card, changing session type to “credit”, it is possible to charge
that pre-paid card without requesting the customer to present card again.

public void transfer_data(marshall_t.msg_transfer_data_t data);

Upon successful Credit-Card / Prepaid read, the information gathered by the VPOS
Is transferred to the peripheral application. See marshall_t.msg_transfer_data_t for more information

public void get_session_status()

Returns the session's status

public void get_reader_state()

Returns the reader's status

public void client_gateway_auth(boolean approved);

The SDK provides means for the peripheral application to provide third-party cashless authorization.
See vmc_configuration_t for more information (mifare_approved_by_vmc_support and mag_card_approved_by_vmc_support)

public long get_transaction_id();

Get the transaction ID - after a credit-card has been presented / read

Socket

T.B.D

General Marshall Facilities

Events

public interface GeneralEvents

void onDCSParamChange(int dcs_param, byte[] data);

void onDisplayEventButtonPressed(byte button_id);

void onDisplayEventTextInput(string input);
void onTransferData(byte[] data);
void onTimeUpdate(
        byte year,
        byte month,
        byte day,
        byte hours,
        byte minutes,
        byte seconds);

API

public void register_callbacks(GeneralEvents events)

register to peripheral general module’s events

public void alert(short id, String msg)

Send alert to DCS. alert will be shown on the DCS, under “Info/Last Alerts”

public void requestTime();

make time request from VPOS. Time response will arrive as an event (onTimeUpdate)

SDK's flags description

public static void main(string[] args)

Initializes all the flags' values:

vmc_config.multi_vend_support = false;

vmc_config.multi_session_support = false;

vmc_config.price_not_final_support = true;

vmc_config.reader_always_on = false;

vmc_config.always_idle = false;

vmc_config.vend_denied_policy = vmc_configuration.vend_denied_policy_cancel;

vmc_config.mifare_approved_by_vmc_support = false;

vmc_config.mag_card_approved_by_vmc_support = false;

vmc_config.dump_packets_level = vmc_configuration.debug_level_dump_moderate; vmc_config.debug = true;

- Most of those flags' values are reflected in the Capabilities bitmap (explained bellow)

multi_vend_support- will the peripheral support multivend (vending of multiple product at the same transaction). Currently only supported with a single session, meaning multisession+ multivend is not yet supported.

- Note: to have multivend working, you need to have it configured in the DCS, under your machine's Payment configurations:

multi_session_support- will enable your device to have multiple sessions happening in parallel. Usually, this flag's value is in correlation to the price_not_final_support's value.

- Note: to have Multisession working, you must configure the following in the DCS, under your machine's configurations:

- MDB
MDB flags- 02 Preselection Enabled

- Payment
Choose product timeout- 0

price_not_final- lets the Nayax device know that the price received from the server is not final (makes the device's Preselection behave like Preauthorization, in order to not have balance captured for a couple of days)

reader_always_on- as the title says, have the device's reader is always turned on.

always_idle- means the transaction starts from the peripheral by the consumer selecting the product (the peripheral can immediately send "Vend request" command without waiting for "Begin Session" trigger).

- Note: this feature must be used if working with the "Display Control" features or for customers that would like to have the authorization of cards to go through their own servers.

vend_denied_policy- irrelevant, N/A.

mifare_approved_by_vmc_support- allows prepaid cards to go through authorization at a 3rd party server. Only relevant if customer specified this need

mag_card approved_by_vmc_support- allows credit cards to go through authorization at a 3rd party server. Only relevant if customer specified this need

Log generation

In order to have logs of the SDK, you need to look at the following flags' value:

dump_packets_level- decides the filtration of the information to be logged. 
Has the following options:

Debug_level_dump_none  =don't generate a log

Debug_level_dump_all = send all of the messages that the SDK generates

Debug_level_dump_moderate = send all of the messages that the SDK generates- except for keep alive and it's ACK (the rest of the ACKs will go though)

debug- decides whether a log file should be made. True- for log generation, False- for disabling log generation

Log location- as written in FileWriter fstream (automatically the path is "c:\\out.txt")

if (vmc_config.debug)

{

BufferedWriter out = null;

try

{

FileWriter fstream = new FileWriter("c:\\out.txt", true); //true tells to append data.

out = new BufferedWriter(fstream);

Log.initialize(out);

} catch (Exception ex)

{

}

}

Java's SDK- simulator manual

Regular transaction (Single session)

how the menu looks:

[0]: Start Vend Session 0

Starts a session (a transaction)

[1]: Cancel Session

Cancels the transaction as if you pressed on Nayax's device X button

[2]: Close Session 0

Only relevant in multisession. Closes the session at the end of a transaction. For example- if you're using an EV charger, and you've finished charging you vehicle- at this point you'll get Close session

[3]: query opened sessions
Relevant from 4.0.0.22 and above- gives information about which sessions are currently opened ("Get Sessions status" command)

[4]: query reader state
Relevant from 4.0.0.22 and above- gives information about whether the reader is enabled (1)/ disabled (0) ("Get Reader status" command)

[5]: cash sale
Simulates cash sale.

[6]: alert
Sends alert to the DCS. Can be seen in Last Alerts

[7]: display message
Irrelevant. Was once used for testing something that no longer exists

[8]: open socket
In order for the peripheral to use Nayax's modem for it's own needs

[9]: close socket
Closes the socket which was opened for the peripheral

[10]: UI control
Tests the Display Control feature. Needs configurations in the DCS

[q]: quit
Quits the simulator program

Transaction flow

Starting a session- [0]: Start Vend Session 0

Present card, let it go through authorization, and wait for "Thank you" screen

Multi session

Multi- session requirements:

vmc_config.multi_session_support = true;

vmc_config.price_not_final_support = true;

how the menu looks:

--- Options ---

[0]: Start Vend Session 0

[1]: Start Vend Session 1

[2]: Cancel Session

[3]: Close Session 0 (will generate "failed to dispense product")

[4]: Close Session 1

[5]: query opened sessions

[6]: query reader state

[7]: cash sale

[8]: alert

[9]: display message

[10]: open socket

[11]: close socket

[12]: UI control

[q]: quit

- Note: Close session 0 causes the product not to be dispensed because of:

m_session.session_status = vmc_vend_t.session_status_fail_to_dispense_e;

While Close session 1 has:

m_session.session_status = vmc_vend_t.session_status_ok_e;

Transaction flow- unsuccessful vend:

Starting a session- [0]: Start Vend Session 0

Present card, let it go through authorization, and wait for "Thank you" screen

Close the session- [3]: Close Session 0

> (+         1ms)        vmc_link: transmitting mdb command: 3

(+         0ms)                : tx:

15:00:01:1a:01:30:00:36:80:13:80:02:05:00:00:00:

01:00:00:00:00:3d:39:

(+         7ms)                : rx:

0a:00:00:1a:00:36:01:30:00:00:8e:22:

(+      1368ms)      marshall_t: received status

(+         1ms)                : rx:

0d:00:01:06:00:36:01:30:0b:04:01:00:00:a6:a0:

(+         0ms)                : tx:

0a:00:00:06:01:30:00:36:00:00:af:0b:

(+         0ms)      vmc_vend_t: received status: 4

(+         0ms)      vmc_vend_t: settlement done

(+         0ms)            --->: settlement: success

(+         0ms)    vmc_socket_t: received status: 4

Would look on the DCS with the following beige color (with 0.00 under Settlement amount):

Transaction flow- successful vend:

Starting a session- [1]: Start Vend Session 1

Present card, let it go through authorization, and wait for "Thank you" screen

Close the session- [4]: Close Session 1

> (+         0ms)        vmc_link: transmitting mdb command: 3

(+         1ms)                : tx:

15:00:01:cf:01:30:00:36:80:13:80:00:05:00:01:00:

01:00:00:00:00:3f:b1:

(+         4ms)                : rx:

0a:00:00:cf:00:36:01:30:00:00:47:1a:

(+      9306ms)      marshall_t: received status

(+         0ms)                : rx:

0d:00:01:0c:00:36:01:30:0b:04:01:00:00:67:34:

(+         0ms)                : tx:

0a:00:00:0c:01:30:00:36:00:00:e1:f8:

(+         2ms)      vmc_vend_t: received status: 4

(+         0ms)      vmc_vend_t: settlement done

(+         0ms)            --->: settlement: success

(+         0ms)    vmc_socket_t: received status: 4

Would look the following way in the DCS:

Checking on session status + Closing/cancelling session

In the (multisession) menu, you've got option number 5. You can check it anytime, not just on powerup. For example in this case I've opened 2 sessions but have not closed them yet:

> (+         0ms)        vmc_link: transmitting mdb command: 3

(+         0ms)                : tx:

0b:00:01:bd:01:30:00:36:80:13:85:a4:ec:

(+         6ms)                : rx:

0a:00:00:bd:00:36:01:30:00:00:c5:fe:

(+         4ms)       mdb_rsp_t: received mdb: -122

(+         0ms)                : rx:

10:00:01:0b:00:36:01:30:80:86:02:00:00:00:01:00:

4c:46:

(+         0ms)                : tx:

0a:00:00:0b:01:30:00:36:00:00:a5:e1:

(+         2ms)      vmc_vend_t: got 2 opened sessions

(+         0ms)            Main: opened session: 0

(+         0ms)            Main: opened session: 1

You can know which sessions you've got opened by reading the Marshall Protocol "Session Status" commad (8.27.5).

Note that we're using little endian, meaning the information I've got is:

0x00 02- number of opened sessions

0x00 00- opened session #1

0x00 01- opened session #2

Multi Vend

Multi- vend requirements:

vmc_config.multi_vend_support = true;

how the menu looks:

--- Options ---

[0]: Start Vend Session 0

[1]: Cancel Session

[2]: Close Session 0 (Only relevant in multisession)

[3]: Paymarket/NOVA Market Session
Simulates the "info session" flow.

[4]: Pay
After using "Info" session, when wanting to charge the consumer (by changing the session's type to “Credit") without requesting the consumer to present card again.
[5]: query opened sessions

[6]: query reader state

[7]: cash sale

[8]: alert

[9]: display message

[10]: open socket

[11]: close socket

[12]: UI control

[q]: quit

Transaction flow

Starting a session- [0]: Start Vend Session 0

Present card, let it go through authorization, and wait for "Thank you" screen

Would look the following way in the DCS:

Capabilities bitmap

This information is being send withing the FirmwareInfo() function (as explained in Marshall Protocol section 8.3). Each of those bits turn on/off another function as following:

| Bit | Description |
| --- | --- |
| 0-5 | Irrelevant for Marshall |
| 6 | Multi session. peripheral works in Multi sessions mode. |
| 7 | Peripheral will approve Contactless proprietary cards. Peripheral is connected to its server and authorizes its proprietary Contactless prepaid cards. NOTE: Once this bit is set, system will support 3rd party proprietary cards ONLY! i.e. there is no possibility to accept other proprietary cards. |
| 8 | Vend request price is not final. Combined with Multi session support bit, enables a Pre-Authorization transaction flow. |
| 9 | Peripheral will approve Magnetic stripe proprietary cards. Peripheral is connected to its server and authorizes its proprietary Magstripe prepaid cards. NOTE: Once this bit is set, system will support 3rd party proprietary cards ONLY! i.e. there is no possibility to accept other proprietary cards. |

For example, if you're working with Multisession+ price not final, the capabilities value would be: 0b0101000000=320 in decimal, and it would show the same in the DCS: